# Intent-Based 5G Network Optimizer with LLM

Agentic AI for Autonomous Optimization of 5G-Advanced Radio Access Networks

A working prototype of an intent-driven network management system built on 3GPP Release 18 principles. Natural language intents are parsed by a Groq-powered LLM agent pipeline and translated into network configurations, while the system continuously monitors KPIs and heals itself autonomously.

Prepared by: Daniyah Aleid

Supervisor: Dr. Abdulaziz AlQabbany

Date: 19-2-2026

# Table of Contents

- 1. Overview

- 2. Architecture

- 3. Agent Pipeline

- 4. Dataset

- 5. Network Topology

- 6. Dynamic Intent Classification

- 7. Multi-Intent Conflict Resolution

- 8. KPI Thresholds

- 9. 3GPP Standards Compliance

- 10. Technology Stack

- 11. Project Structure

- 12. References

# 1. Overview

Traditional network management requires operators to manually translate business goals into low-level RAN parameters. This system eliminates that gap:

An operator types: "Prioritize emergency communications at the hospital now"

Six AI agents process the request end-to-end

The network is reconfigured, monitored, and self-healed — no manual steps

Key Architectural Principle: LLM translates, rules validate, policies execute. The LLM is never given direct control over the network; it only parses intent and proposes configurations. A deterministic rule-based validator acts as the safety gate before any action is taken.

# 2. Architecture

The system is organized into three layers:

## Layer 1: Streamlit UI

Tab 1: Intent Processing

Tab 2: Network Monitor + Topology

Tab 3: Multi-Intent Conflict Resolution

## Layer 2: CrewAI Agent Pipeline (Groq LLM)

Agent 1: Intent Parser (LLM)

Agent 2: Reasoner (LLM)

Agent 3: Validator (Rule-based) ← Safety gate

Agent 4: Planner (LLM)

Agent 5: Monitor (LLM)

Agent 6: Optimizer (LLM)

## Layer 3: Network Simulator + Real Dataset

6G HetNet CSV • 5,000 rows • 49 Cell_IDs

Dataset-driven simulator cycles through real records sequentially

# 3. Agent Pipeline

All agents except the Validator are powered by Groq LLM. The LLM dynamically generates intent classifications, clarifying questions, and network configurations — nothing is hardcoded. Every LLM output is safety-clamped before execution.

| # | Agent | Powered By | Role |
| --- | --- | --- | --- |
| 1 | Intent Parser | Groq LLM | Converts natural language into a structured intent object. Creates dynamic intent types (e.g. stadium_event, drone_delivery) — not limited to a fixed list. |
| 2 | Reasoner | Groq LLM | Generates context-aware clarifying questions, assesses feasibility, identifies risks, simulates impact |
| 3 | Validator | Rule-based | Hard safety gate — enforces bandwidth caps, user limits, confidence thresholds. Cannot be overridden by LLM |
| 4 | Planner | Groq LLM | LLM generates full 3GPP config (slice, bandwidth, latency, MIMO, scheduler, cells) based on the intent. All values safety-clamped. |
| 5 | Monitor | Groq LLM | Reads real KPIs from the dataset/simulator, detects anomalies, triggers alerts |
| 6 | Optimizer | Groq LLM | Executes corrective actions, with automatic rollback if metrics degrade |

## Why Agent 3 is Rule-Based

The Validator enforces hard limits that must be deterministic and auditable:

| Limit | Value | Reason |
| --- | --- | --- |
| Max bandwidth | 500 Mbps | Physical capacity |
| Min bandwidth | 10 Mbps | Minimum viable allocation |
| Min confidence | 70% | Reject uncertain LLM output |
| Max cells active at once | 5 | Prevent mass activation |
| Min active cells | 5 | Maintain coverage |

LLMs are non-deterministic. Safety constraints must not be.

# 4. Dataset

File: data/6G_HetNet_Transmission_Management.csv

| Property | Value |
| --- | --- |
| Rows | 5,000 |
| Columns | 24 |
| Unique Cell_IDs | 49 (range 1–49) |
| Cell types | Macro (10), Micro (17), Pico (8), Femto (14) |

## Key Columns Used

| Column | Used For |
| --- | --- |
| Cell_ID | Pinning topology cells to specific real cells |
| Cell_Type | Macro / Micro / Pico / Femto classification |
| Achieved_Throughput_Mbps | Live KPI display |
| Network_Latency_ms | SLA monitoring |
| Resource_Utilization | Cell load % |
| Signal_to_Noise_Ratio_dB | RAN quality metric |
| Interference_Level_dB | Interference monitoring |
| QoS_Satisfaction | Service quality score |
| Packet_Loss_Ratio | Reliability metric |

## How the Dataset is Used

Simulator: Simulator reads one row per get_metrics() call, cycling through all 5,000 records sequentially

Topology: Each of the 12 HetNet topology cells is pinned to a dedicated Cell_ID so every cell displays real, distinct KPI data from its own real-world counterpart

# 5. Network Topology

The UI renders a 12-cell Heterogeneous Network (HetNet) with 4 cell tiers. Each topology cell is mapped to a specific Cell_ID in the real dataset:

| Cell | Type | Dataset Cell_ID | Real Rows |
| --- | --- | --- | --- |
| C01 Central | Macro | 3 | 103 |
| C02 North | Macro | 4 | 108 |
| C03 South | Macro | 13 | 98 |
| C04 Micro-1 | Micro | 2 | 99 |
| C05 Micro-2 | Micro | 7 | 78 |
| C06 Micro-3 | Micro | 12 | 99 |
| C07 Micro-4 | Micro | 14 | 94 |
| C08 Pico-1 | Pico | 9 | 98 |
| C09 Pico-2 | Pico | 11 | 102 |
| C10 Pico-3 | Pico | 18 | 95 |
| C11 Femto-1 | Femto | 1 | 102 |
| C12 Femto-2 | Femto | 5 | 101 |

Cell health (healthy / warning / critical) is computed from real throughput, latency, and load values.

# 6. Dynamic Intent Classification

The system uses the Groq LLM to classify user intents into ANY descriptive type — there is no hardcoded whitelist. The LLM generates a custom snake_case intent type that best describes the user's request (e.g. stadium_event, drone_delivery, smart_agriculture, world_cup_event). This means the system can handle novel use cases without code changes.

## Slice Type Assignment

The 3GPP slice type (eMBB, URLLC, mMTC) is determined by keyword matching on the intent type name, so even custom LLM-generated types get an appropriate slice:

| Slice | Keyword Triggers | Use Cases |
| --- | --- | --- |
| URLLC | emergency, healthcare, surgery, vehicle, drone, factory, safety, robot, control | Low latency, high reliability |
| mMTC | iot, sensor, agriculture, energy, grid, meter, scada, device | Massive device count, low bandwidth per device |
| eMBB | Everything else (default) | High bandwidth — streaming, events, gaming, general |

## Example Intent Classifications

| User Input | LLM-Generated Type | Slice |
| --- | --- | --- |
| "Optimize for the football match tonight" | stadium_event | eMBB |
| "Emergency at the hospital" | emergency | URLLC |
| "10,000 IoT sensors in the factory" | iot_deployment | mMTC |
| "Drone delivery fleet management" | drone_delivery | URLLC |
| "World Cup match tonight" | world_cup_event | eMBB |
| "Smart agriculture monitoring" | smart_agriculture | mMTC |
| "Improve network performance" | general_optimization | eMBB |

Note: The LLM estimates expected users, bandwidth, latency, and priority based on the use case context. For example, a stadium event may estimate 50,000 users while an emergency may estimate 200 responders.

# 7. Multi-Intent Conflict Resolution

When multiple stakeholders submit competing intents simultaneously, the system resolves conflicts by:

Ranking intents by priority (Emergency > Healthcare > Stadium > General)

Allocating bandwidth from a shared 500 Mbps pool

Identifying resource conflicts (overlapping cells, spectrum)

Producing a negotiated configuration that satisfies as many SLAs as possible

# 8. KPI Thresholds

| KPI | Target | Warning | Critical |
| --- | --- | --- | --- |
| Latency | < 50 ms | < 80 ms | ≥ 100 ms |
| Throughput | > 100 Mbps | > 60 Mbps | ≤ 30 Mbps |
| Packet Loss | < 0.01% | < 0.1% | ≥ 1.0% |
| Cell Load | < 70% | < 85% | ≥ 95% |

# 9. 3GPP Standards Compliance

| Standard | Relevance |
| --- | --- |
| 3GPP TS 28.312 | Intent-Based Management |
| 3GPP TS 38.843 | AI/ML for RAN |
| 3GPP TS 28.104 | Self-Optimization (SON) |
| 3GPP Release 18 | 5G-Advanced specifications |
| TM Forum IG1230 | Autonomous Networks (Level 5) |

# 10. Technology Stack

| Component | Technology |
| --- | --- |
| UI | Streamlit |
| Agent Framework | CrewAI 0.86.0 |
| LLM | Groq — Llama 3.3 70B Versatile |
| LLM Router | LiteLLM (via CrewAI) |
| Visualization | Plotly |
| Data | Pandas / NumPy |
| Language | Python 3.10+ |

# 11. Project Structure

| File / Folder | Description |
| --- | --- |
| app.py | Streamlit UI — main entry point |
| requirements.txt | Python dependencies |
| .env.example | Environment variable template |
| agents/crew.py | CrewAI crew definition + Groq LLM config |
| agents/llm_client.py | LangChain-Groq client for tool-level LLM calls |
| agents/intent_agent.py | Intent Parser agent definition |
| agents/planner_agent.py | Planner agent definition |
| agents/monitor_agent.py | Monitor agent definition |
| agents/optimizer_agent.py | Optimizer agent definition |
| tools/intent_tools.py | parse_intent (LLM + keyword fallback) |
| tools/config_tools.py | LLM-generated 3GPP configs with safety clamping |
| tools/monitor_tools.py | get_metrics, check_status |
| tools/action_tools.py | execute_action, rollback |
| tools/reasoning_llm.py | LLM-generated clarifying questions + conflict narration |
| simulator/network_sim.py | Dataset-driven 5G network simulator |
| config/settings.py | LLM config, KPI thresholds, network limits |
| data/6G_HetNet_Transmission_Management.csv | Real dataset (5,000 rows) |
| data/templates.json | 3GPP slice configuration templates |

# 12. References

3GPP TS 28.312 — Intent driven management services for mobile networks

3GPP Release 18 — 5G-Advanced specifications

TM Forum IG1230 — Autonomous Networks Technical Architecture

ETSI ZSM — Zero-touch network and Service Management